Lab - Configure Administrative Roles

Topology

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R1 | G0/0/1 | 192.168.1.1 | 255.255.255.0 | N/A | S1 F0/5 |
| R2 | G0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
| R2 | G0/0/1 | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| R3 | G0/0/0 | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| R3 | G0/0/1 | 192.168.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S1 F0/6 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | S3 F0/18 |

Blank Line, No additional information

Objectives

Part 1: Configure Basic Device Settings

Cable the network as shown in the topology.

Configure basic IP addressing for routers and PCs.

Configure OSPF routing.

Configure PC hosts.

Verify connectivity between hosts and routers.

Part 2: Configure Administrative Roles

Create multiple role views and grant varying privileges.

Verify and contrast views.

Background / Scenario

The router is a critical component in any network. It controls the movement of data into and out of the network and between devices within the network. It is particularly important to protect network routers because the failure of a routing device could make sections of the network, or the entire network, inaccessible. Controlling access to routers and enabling reporting on routers is critical to network security and should be part of a comprehensive security policy.

In this lab, you will build a multi-router network and configure the routers and hosts. You will configure administrative roles with different privilege levels.

Note: The routers used with hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.6 (universalk9 image). The switches used in the labs are Cisco Catalyst 2960+ with Cisco IOS Release 15.2(7) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Before you begin, ensure that the routers and the switches have been erased and have no startup configurations.

Required Resources

3 Routers (Cisco 4221 with Cisco XE Release 16.9.6 universal image or comparable with a Security Technology Package license)

2 Switches (Cisco 2960+ with Cisco IOS Release 15.2(7) lanbasek9 image or comparable)

2 PCs (Windows OS with a terminal emulation program, such as PuTTY or Tera Term installed)

Console cables to configure Cisco networking devices

Ethernet cables as shown in the topology

Instructions

Configure Basic Device Settings

In this part, set up the network topology and configure basic settings, such as interface IP addresses.

Cable the network.

Attach the devices, as shown in the topology diagram, and cable as necessary.

Configure basic settings for each router.

Open configuration window

Console into the router and enable privileged EXEC mode.

Router> enable

Router# configure terminal

Configure host names as shown in the topology.

R1(config)# hostname R1

Configure interface IP addresses as shown in the IP Addressing Table.

R1(config)# interface g0/0/0

R1(config-if)# ip address 10.1.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config)# interface g0/0/1

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

To prevent the router from attempting to translate incorrectly entered commands as though they were host names, disable DNS lookup. R1 is shown here as an example.

R1(config)# no ip domain-lookup

Configure OSPF routing on the routers.

Use the router ospf command in global configuration mode to enable OSPF on R1.

R1(config)# router ospf 1

Configure the network statements for the networks on R1. Use an area ID of 0.

R1(config-router)# network 192.168.1.0 0.0.0.255 area 0

R1(config-router)# network 10.1.1.0 0.0.0.3 area 0

Configure OSPF on R2 and R3.

R2(config)# router ospf 1

R2(config-router)# network 10.1.1.0 0.0.0.3 area 0

R2(config-router)# network 10.2.2.0 0.0.0.3 area 0

R3(config)# router ospf 1

R3(config-router)# network 10.2.2.0 0.0.0.3 area 0

R3(config-router)# network 192.168.3.0 0.0.0.255 area 0

Issue the passive-interface command to change the G0/0/1 interface on R1 and R3 to passive.

R1(config)# router ospf 1

R1(config-router)# passive-interface g0/0/1

R3(config)# router ospf 1

R3(config-router)# passive-interface g0/0/1

Verify OSPF neighbors and routing information.

Issue the show ip ospf neighbor command to verify that each router lists the other routers in the network as neighbors.

R1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

10.2.2.2          1   FULL/BDR        00:00:37    10.1.1.2        GigabitEthernet0/0/0

Issue the show ip route command to verify that all networks display in the routing table on all routers.

R1# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        10.1.1.0/30 is directly connected, GigabitEthernet0/0/0

L        10.1.1.1/32 is directly connected, GigabitEthernet0/0/0

O        10.2.2.0/30 [110/2] via 10.1.1.2, 00:01:11, GigabitEthernet0/0/0

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.1.0/24 is directly connected, GigabitEthernet0/0/1

L        192.168.1.1/32 is directly connected, GigabitEthernet0/0/1

O     192.168.3.0/24 [110/3] via 10.1.1.2, 00:01:07, GigabitEthernet0/0/0

Configure PC host IP settings.

Configure a static IP address, subnet mask, and default gateway for PC-A and PC-C as shown in the IP Addressing Table.

Verify connectivity between PC-A and PC-C.

Ping from R1 to R3.

If the pings are not successful, troubleshoot the basic device configurations before continuing.

Ping from PC-A, on the R1 LAN, to PC-C, on the R3 LAN.

If the pings are not successful, troubleshoot the basic device configurations before continuing.

Note: If you can ping from PC-A to PC-C you have demonstrated that OSPF routing is configured and functioning correctly. If you cannot ping but the device interfaces are up and IP addresses are correct, use the show run, show ip ospf neighbor, and show ip route commands to help identify routing protocol-related problems.

Save the basic running configuration for each router.

Save the basic running configuration for the routers as text files on your PC. These text files can be used to restore configurations later in the lab.

Close configuration window

Configure Administrative Roles

In this part of lab, you will:

Create multiple administrative roles, or views, on routers R1 and R3.

Grant each view varying privileges.

Verify and contrast the views.

The role-based CLI access feature allows the network administrator to define views, which are a set of operational commands and configuration capabilities that provide selective or partial access to Cisco IOS EXEC and configuration (config) mode commands. Views restrict user access to the Cisco IOS CLI and configuration information. A view can define which commands are accepted and what configuration information is visible.

Note: Perform all tasks on both R1 and R3. The procedures and output for R1 are shown here.

If an administrator wants to configure another view to the system, the system must be in root view. When a system is in root view, the user has the same access privileges as a user who has level-15 privileges, but the root view user can also configure a new view and add or remove commands from the view. When you are in a CLI view, you have access only to the commands that have been added to that view by the root view user.

Enable AAA on router R1.

Open configuration window

To define views, enable AAA on the router.

R1# configure terminal

R1(config)# aaa new-model

Configure privileged EXEC mode password.

A privileged EXEC mode password is required to access the root view. The password cisco12345 is used in this example.

R1(config)# enable secret cisco12345

R1# exit

Enable the root view.

Use the command enable view to enable the root view.

R1# enable view

Password: cisco12345

Create the admin1 view, establish a password, and assign privileges.

The admin1 user is the top-level user below root that is allowed to access this router. It has the most authority. The admin1 user can use all show, config, and debug commands. Use the following command to create the admin1 view while in the root view.

R1# configure terminal

R1(config)# parser view admin1

R1(config-view)#

Note: To delete a view, use the command no parser view viewname.

Associate the admin1 view with an encrypted password.

R1(config-view)# secret admin1pass

R1(config-view)#

Review the commands that can be configured in the admin1 view. Use the commands ? command to see available commands. The following is a partial listing of the available commands.

R1(config-view)# commands ?

RITE-profile           Router IP traffic export profile command mode

RMI Node Config        Resource Policy Node Config mode

RMI Resource Group     Resource Group Config mode

RMI Resource Manager   Resource Manager Config mode

RMI Resource Policy    Resource Policy Config mode

SASL-profile           SASL profile configuration mode

aaa-attr-list          AAA attribute list config mode

aaa-user               AAA user definition

accept-dialin          VPDN group accept dialin configuration mode

accept-dialout         VPDN group accept dialout configuration mode

address-family         Address Family configuration mode

<output omitted>

Add all config, show, and debug commands to the admin1 view and then exit from view configuration mode.

R1(config-view)# commands exec include all show

R1(config-view)# commands exec include all config terminal

R1(config-view)# commands exec include all debug

R1(config-view)# end

Verify the admin1 view.

R1# enable view admin1

Password: admin1pass

R1# show parser view

Current view is ‘admin1’

Examine the commands available in the admin1 view.

R1# ?

Exec commands:

<0-0>/<0-4>  Enter card slot/sublot number

configure    Enter configuration mode

debug        Debugging functions (see also 'undebug')

do-exec      Mode-independent "do-exec" prefix support

enable       Turn on privileged commands

exit         Exit from the EXEC

show         Show running system

Note: There may be more EXEC commands available than are displayed. This depends on your device and the IOS image used.

Examine the show commands available in the admin1 view.

R1# show ?

aaa                       Show AAA values

access-expression         List access expression

access-lists              List access lists

acircuit                  Access circuit info

adjacency                 Adjacent nodes

aliases                   Display alias commands

alignment                 Show alignment information

appfw                     Application Firewall information

archive                   Archive functions

arp                       ARP table

<output omitted>

Create the admin2 view, establish a password, and assign privileges.

The admin2 user is a junior administrator in training who is allowed to view all configurations but is not allowed to configure the routers or use debug commands.

Use the enable view command to enable the root view, and enter the enable secret password cisco12345.

R1# enable view

Password: cisco12345

Use the following command to create the admin2 view.

R1# configure terminal

R1(config)# parser view admin2

Associate the admin2 view with a password.

R1(config-view)# secret admin2pass

Add all show commands to the view, and then exit from view configuration mode.

R1(config-view)# commands exec include all show

R1(config-view)# end

Verify the admin2 view.

R1# enable view admin2

Password: admin2pass

R1# show parser view

Current view is ‘admin2’

Examine the commands available in the admin2 view.

R1# ?

Exec commands:

<0-0>/<0-4>  Enter card slot/sublot number

do-exec      Mode-independent "do-exec" prefix support

enable       Turn on privileged commands

exit         Exit from the EXEC

show         Show running system information

Note: There may be more EXEC commands available than are displayed. This depends on your device and the IOS image used.

Question:

What is missing from the list of admin2 commands that is present in the admin1 commands?

Type your answers here.

Create the tech view, establish a password, and assign privileges.

The tech user typically installs end-user devices and cabling. Tech users are only allowed to use selected show commands.

Use the enable view command to enable the root view, and enter the enable secret password cisco12345.

R1# enable view

Password: cisco12345

Use the following command to create the tech view.

R1(config)# parser view tech

Associate the tech view with a password.

R1(config-view)# secret techpasswd

Add the following show commands to the view and then exit from view configuration mode.

R1(config-view)# commands exec include show version

R1(config-view)# commands exec include show interfaces

R1(config-view)# commands exec include show ip interface brief

R1(config-view)# commands exec include show parser view

R1(config-view)# end

Verify the tech view.

R1# enable view tech

Password: techpasswd

R1# show parser view

Current view is ‘tech’

Examine the commands available in the tech view.

R1# ?

Exec commands:

<0-0>/<0-4>  Enter card slot/sublot number

do-exec      Mode-independent "do-exec" prefix support

enable       Turn on privileged commands

exit         Exit from the EXEC

show         Show running system information

Note: There may be more EXEC commands available than are displayed. This depends on your device and the IOS image used.

Examine the show commands available in the tech view.

R1# show ?

banner      Display banner information

flash0:     display information about flash0: file system

flash1:     display information about flash1: file system

flash:      display information about flash: file system

interfaces  Interface status and configuration

ip          IP information

parser      Display parser information

usbflash0:  display information about usbflash0: file system

version     System hardware and software status

Note: There may be more EXEC commands available than are displayed. This depends on your device and the IOS image used.

Issue the show ip interface brief command.

Question:

Were you able to do it as the tech user? Explain.

Yes, as the tech user, I was able to run the commands that were explicitly added to the tech view, such as: show version, show interfaces show ip interface brief, show parser view.

This worked because, in the tech view configuration, only these specific show commands were granted access.

Any commands not included in the tech view, like show ip route or configuration commands, would be denied because role-based views restrict the user to only the commands explicitly allowed.

Issue the show ip route command.

Question:

Were you able to do it as the tech user?

No, as the tech user, I was not able to run the show ip route command because it was not included in the list of allowed commands when the tech view was created.

Return to root view with the enable view command.

R1# enable view

Password: cisco12345

Issue the show run command to see the views you created.

Question:

For tech view, why are the show and show ip commands listed as well as show ip interface and show ip interface brief?

For the tech view, the commands show and show ip appear because when you explicitly allow commands like show ip interface or show ip interface brief, Cisco IOS automatically includes the parent commands (show and show ip) in the list.

This happens because:

show is the base command for all show commands.

show ip is the base command for all IP-related show commands.

Granting access to a subcommand also requires access to its parent commands, so they get listed automatically.

Configure the same administrative roles on router R3.

Save the configuration on routers R1 and R3.

Save the running configuration to the startup configuration from the privileged EXEC prompt.

Close configuration window

- Router Interface Summary Table

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 4221 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 4300 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |

Blank Line, No additional information

Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface.

Screenshot

end of document